How to Program an ATA PTX5 Garage Door Remote

I recommend that you first read the instructions to ensure you have a clear understanding of the process.

MOTORS WITH A “MODE” BUTTON

- Press and release the “MODE” button on the motor repeatedly until the LED on the transmitter lights up.

- Press and hold the blue down arrow button on the motor until Step 5.

- On the transmitter, press and hold button 1 for 2 seconds.

- Press and hold button 1 again for 2 seconds.

- Release the blue down arrow button on the motor.

- Press and release the “MODE” button on the motor.

MOTORS WITH A “DOOR CODE” BUTTON

- Press and hold the “DOOR CODE” button on the motor until Step 4.

- Press and hold button 1 on the remote for 2 seconds.

- Press and hold button 1 again for 2 seconds.

- Release the “DOOR CODE” button on the motor.

0458 910 752

sales@remotebloke.com.au