How to Program an Avanti Garage Door Remote

I recommend that you first read the instructions to ensure you have a clear understanding of the process.

- Locate the Learn/Code set button on the motor. This is usually under the lamp cover or on the back of the motor.

- Press the Learn/Code set button once and immediately release the button. A light on the motor will illuminate.

- Press button #1 on the new remote for 2 seconds. The light on the motor will extinguish.

- Press button #1 for 2 seconds again. The lights on the motor will begin to flash rapidly.

- Once the light stops flashing, test your new remote.

- Your new remote is now coded.

If the remote doesn't successfully code to the motor or the motor displays a continuous flashing light, it's likely the learn button was held down for too long.

To fix this, open and close the garage door 4 times using an existing remote. Once that's done, try coding the new remote again.

0458 910 752

sales@remotebloke.com.au