How to Program a Steeline ZT-07 Garage Door Remote

I recommend that you first read the instructions to ensure you have a clear understanding of the process.

- Press and hold the “P” and “+” buttons on the motor until all lights on the motor turn on and LED 7 flashes.

- Press “+” button on the motor once. All LED lights turn off except LED 7.

- Quickly press and release the required push button on your new remote twice. All LED lights turn on and LED 7 will begin flashing.

- Press the “P” button once on the motor to save. LED3 will switch on indicating that the remote has been successfully programmed.

- Test the operation of your new remote.

- Your remote has now been coded.

0458 910 752

sales@remotebloke.com.au